Program and Branch Name:        B.Tech   in  CSE TWIN (3RD SEMESTER)

PBL Progress Report – II

| Field | Details |
| --- | --- |
| Course Name | Computer Architecture & Parallel Processing |
| Course Code | CCSE 0304 |
| Faculty Name | Dr. AMRITA BHATNAGAR |
| Assignment Type | Group Assignment |
| Student(s) Name | - Akshya Gupta - Anshika Verma - Priyanshu Rai - Kajal Tiwari |
| ERP ID (I Year) / Roll No. (II Year onwards) | - 2501331930010 - 2501331930014 - 2501331930030 - 2501331930038 ALL 2ND YEAR |
| Team/Group No. (Group/Capstone only) | GROUP - 02 |
| Problem Statement / Project Title | Topic 84: Integrated Smart City Computing Framework |
| SDG(s) | SDG 11: Sustainable Cities and Communities |
| Reporting Month | Month 2 |
| Date of Submission | 08 OCTOBER 2026 |

Student Declaration

I/We certify that this assignment is my/our own work, with all sources appropriately acknowledged. I/We understand that a 0% mark and further academic penalties may apply if the content is found to be plagiarised or inappropriately AI-generated.

Student Name & Signature(s):      AKSHYA GUPTA

ANSHIKA VERMA

KAJAL TIWARI

PRIYANSHU RAI

Date:   08 OCTOBER 2026

PBL Progress Report – II

Review 2: Development & Application

Review Coverage:

- Individual / Group Assignment: Unit/Module 3 and Unit/Module 4

- Capstone Project: Rubric Categories 3 and 4

| Field | Details |
| --- | --- |
| 1. Previous Review Feedback | Review 1 approved the bus arbiter and Booth's ALU. The faculty instructed the team to provide concrete simulation evidence of: - Zero-stall instruction pipeline execution during recurring sensor loops. - Cache coherence state transitions to prevent dirty reads across municipal data streams. |
| 2. Action Taken on Previous Feedback | - Built a cycle-accurate 5-stage RISC pipeline in `pipeline.py` with hardware operand forwarding. - Built a 4-state MESI directory coherence engine in `cache_numa.py` to eliminate bus-snooping saturation. - Verified all cycle behaviors using a native Python testbench (`main.py`) and an interactive GUI (`gui_dashboard.py`). |
| 3. Research / Data Collection Progress | - Hennessy & Patterson (6th ed.): Verified that EX-to-EX forwarding eliminates Read-After-Write (RAW) data stalls. - Sorin, Hill, & Wood: Confirmed that directory-based MESI protocols scale linearly without flooding shared interconnects. - Municipal Telemetry Profiling: Extracted streaming rates from traffic and utility sensors to construct realistic read/write test vectors. |
| 4. Unit/Module Concepts Applied | Module 3: Control Unit & Pipelining - Classical 5-Stage RISC Pipeline: Instruction Fetch (IF), Decode (ID), Execute (EX), Memory (MEM), Write-Back (WB). - Hardware Forwarding Unit: Resolves RAW data hazards by routing ALU results directly from EX/MEM back to EX. - Dynamic Branch Prediction: 2-bit saturating counter to prevent pipeline flushes during periodic sensor-polling loops. - Hardwired Control Logic: Combinational logic prioritized for minimal single-cycle latency. Module 4: Memory Hierarchy & Cache Coherence - Split L1 Cache: Separate 32 KB Instruction and 32 KB Data caches to eliminate structural memory hazards. - 4-State MESI Protocol: Modified (M), Exclusive (E), Shared (S), Invalid (I). - Directory-Based Invalidation: Tracks node sharers and invalidates stale caches point-to-point without bus broadcast storming. - Latency Modeling: 1-cycle L1 local cache hit vs. multi-cycle remote memory fetches. |
| 5. Methodology / Approach Followed | - Step 1: Modeled the 5-stage RISC pipeline datapath and forwarding comparators in `pipeline.py`. - Step 2: Implemented the directory tracking bitmask and MESI state-transition rules in `cache_numa.py`. - Step 3: Integrated Modules 1–5 into `main.py` with a back-to-back dependency test (`LOAD` followed by dependent `ADD`). - Step 4: Developed `gui_dashboard.py` (Tkinter) to inspect clock cycles, pipeline registers, and cache tags in real time. |
| 6. Work Completed During Month 2 | - Fully coded and verified Modules 3 and 4 in Python. - Confirmed zero stall bubbles during RAW hazard execution using EX-to-EX forwarding. - Confirmed MESI transitions: Cold Read (I -> E), Shared Read (E -> S), Write (S -> I across sharers, M on writer). - Achieved a measured 60.0% Multi-Core Cache Hit Rate. - Updated project presentation slides and assembled simulation evidence. |
| 7. Analysis / Development / Implementation | - Pipeline Forwarding Analysis: An unforwarded pipeline stalls for 2 clock cycles on `LOAD R1` -> `ADD R2, R1, R3`. Our forwarding unit forwards the loaded/computed operand into the EX stage at Cycle 4, achieving 0 stall bubbles. - Cache Invalidation Analysis: When Node 0 and Node 1 share Tag 0x3F2A (State S) and Node 2 writes to it, the directory sends invalidations exclusively to Nodes 0 and 1 (transitioning them to State I) and grants Node 2 State M ownership. Stale reads are eliminated. |
| 8. Results / Interim Outcomes | - Pipeline Performance: Clock-per-Instruction (CPI) reduced to ~1.06 for telemetry-polling routines. - Bus Invalidation Efficiency: Only 2 targeted messages generated on writes, preserving interconnect bandwidth. - Cache Efficiency: 60.0% Cache Hit Rate verified across concurrent multi-node test sequences. - Access Speed: 1-cycle L1 hit access verified for local telemetry reads. |
| 9. Evidence / Supporting Material | - Terminal logs from `main.py` showing cycle-by-cycle pipeline progression and directory invalidations. - Dashboard screenshot from `gui_dashboard.py` showing live clock stepping and register state changes. - MESI state-transition table and pipeline forwarding schematics. |
| 10. Challenges & Corrective Actions | - Challenge 1: Initial test scripts yielded a 0.0% hit rate because requests were purely cold compulsory misses. Solution: Added temporal locality reads to the same address tag, raising the verified hit rate to 60.0%. - Challenge 2: Simultaneous instruction fetches and data memory accesses created structural hazards. Solution: Adopted a Harvard-style split L1 architecture (32 KB I-Cache + 32 KB D-Cache). |
| 11. Individual / Team Contribution | - Kajal Tiwari (2501331930030): Led Module 3; implemented the 5-stage RISC pipeline and operand forwarding logic. - Anshika Verma (2501331930014): Led Module 4; designed the MESI directory protocol and evaluated hit/miss latencies. - Akshya Gupta (2501331930010): Team Lead & Integrator; embedded the Booth ALU into the EX stage, wired the memory stage to the MESI cache, and built the visualizer. - Priyanshu Rai (2501331930038): Interfaced the Module 1 preemptive bus arbiter with the memory subsystem for sensor DMA bursts. |
| 12. Overall Progress (%) | 75% (Modules 1, 2, 3, and 4 are complete and validated; remaining work focuses on final Module 5 multi-node mesh scaling and report synthesis). |
| 13. Plan for Final Review | - Finalize Module 5: Benchmark 2D-Mesh X-Y dimension-order routing latencies across 16-node arrays. - Draft the comprehensive 20–25 page final project documentation. - Prepare the final team presentation and live dashboard demonstration for the viva voce. ashboard demonstration for the viva voce. |